This electronic document was downloaded from the GPO web site, May 2004, and is provided for information purposes only. The Code of Federal Regulations, Title 21, is updated each year in early summer. The most current version of the regulations may be found at the GPO web site or from the current printed version.

(This document also available in PDF and Spanish)

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.3]

[Page 215-216]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart A--General Provisions

Sec. 110.3  Definitions.

The definitions and interpretations of terms in section 201 of the

Federal Food, Drug, and Cosmetic Act (the act) are applicable to such

terms when used in this part. The following definitions shall also

apply:

(a) Acid foods or acidified foods means foods that have an

equilibrium pH of 4.6 or below.

(b) Adequate means that which is needed to accomplish the intended

purpose in keeping with good public health practice.

(c) Batter means a semifluid substance, usually composed of flour

and other ingredients, into which principal components of food are

dipped or with which they are coated, or which may be used directly to

form bakery foods.

(d) Blanching, except for tree nuts and peanuts, means a

prepackaging heat treatment of foodstuffs for a sufficient time and at a

sufficient temperature to partially or completely inactivate the

naturally occurring enzymes and to effect other physical or biochemical

changes in the food.

(e) Critical control point means a point in a food process where

there is a high probability that improper control may cause, allow, or

contribute to a hazard or to filth in the final food or decomposition of

the final food.

(f) Food means food as defined in section 201(f) of the act and

includes raw materials and ingredients.

(g) Food-contact surfaces are those surfaces that contact human food

and those surfaces from which drainage onto the food or onto surfaces

that contact the food ordinarily occurs during the normal course of

operations. ``Food-contact surfaces'' includes utensils and food-contact

surfaces of equipment.

(h) Lot means the food produced during a period of time indicated by

a specific code.

(i) Microorganisms means yeasts, molds, bacteria, and viruses and

includes, but is not limited to, species having public health

significance. The term ``undesirable microorganisms'' includes those

microorganisms that are of public health significance, that subject food

to decomposition, that indicate that food is contaminated with filth, or

that otherwise may cause food to be adulterated within the meaning of

the act. Occasionally in these regulations, FDA used the adjective

``microbial'' instead of using an adjectival phrase containing the word

microorganism.

(j) Pest refers to any objectionable animals or insects including,

but not limited to, birds, rodents, flies, and larvae.

[[Page 216]]

(k) Plant means the building or facility or parts thereof, used for

or in connection with the manufacturing, packaging, labeling, or holding

of human food.

(l) Quality control operation means a planned and systematic

procedure for taking all actions necessary to prevent food from being

adulterated within the meaning of the act.

(m) Rework means clean, unadulterated food that has been removed

from processing for reasons other than insanitary conditions or that has

been successfully reconditioned by reprocessing and that is suitable for

use as food.

(n) Safe-moisture level is a level of moisture low enough to prevent

the growth of undesirable microorganisms in the finished product under

the intended conditions of manufacturing, storage, and distribution. The

maximum safe moisture level for a food is based on its water activity

(aw). An aw will be considered safe for a food if

adequate data are available that demonstrate that the food at or below

the given aw will not support the growth of undesirable

microorganisms.

(o) Sanitize means to adequately treat food-contact surfaces by a

process that is effective in destroying vegetative cells of

microorganisms of public health significance, and in substantially

reducing numbers of other undesirable microorganisms, but without

adversely affecting the product or its safety for the consumer.

(p) Shall is used to state mandatory requirements.

(q) Should is used to state recommended or advisory procedures or

identify recommended equipment.

(r) Water activity (aw) is a measure of the free moisture

in a food and is the quotient of the water vapor pressure of the

substance divided by the vapor pressure of pure water at the same

temperature.

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.5]

[Page 216]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart A--General Provisions

Sec. 110.5  Current good manufacturing practice.

(a) The criteria and definitions in this part shall apply in

determining whether a food is adulterated (1) within the meaning of

section 402(a)(3) of the act in that the food has been manufactured

under such conditions that it is unfit for food; or (2) within the

meaning of section 402(a)(4) of the act in that the food has been

prepared, packed, or held under insanitary conditions whereby it may

have become contaminated with filth, or whereby it may have been

rendered injurious to health. The criteria and definitions in this part

also apply in determining whether a food is in violation of section 361

of the Public Health Service Act (42 U.S.C. 264).

(b) Food covered by specific current good manufacturing practice

regulations also is subject to the requirements of those regulations.

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.10]

[Page 216-217]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart A--General Provisions

Sec. 110.10  Personnel.

The plant management shall take all reasonable measures and

precautions to ensure the following:

(a) Disease control. Any person who, by medical examination or

supervisory observation, is shown to have, or appears to have, an

illness, open lesion, including boils, sores, or infected wounds, or any

other abnormal source of microbial contamination by which there is a

reasonable possibility of food, food-contact surfaces, or food-packaging

materials becoming contaminated, shall be excluded from any operations

which may be expected to result in such contamination until the

condition is corrected. Personnel shall be instructed to report such

health conditions to their supervisors.

(b) Cleanliness. All persons working in direct contact with food,

food-contact surfaces, and food-packaging materials shall conform to

hygienic practices while on duty to the extent necessary to protect

against contamination of food. The methods for maintaining cleanliness

include, but are not limited to:

(1) Wearing outer garments suitable to the operation in a manner

that protects against the contamination of food, food-contact surfaces,

or food-packaging materials.

(2) Maintaining adequate personal cleanliness.

(3) Washing hands thoroughly (and sanitizing if necessary to protect

against contamination with undesirable microorganisms) in an adequate

hand-washing facility before starting work, after each absence from the

work

[[Page 217]]

station, and at any other time when the hands may have become soiled or

contaminated.

(4) Removing all unsecured jewelry and other objects that might fall

into food, equipment, or containers, and removing hand jewelry that

cannot be adequately sanitized during periods in which food is

manipulated by hand. If such hand jewelry cannot be removed, it may be

covered by material which can be maintained in an intact, clean, and

sanitary condition and which effectively protects against the

contamination by these objects of the food, food-contact surfaces, or

food-packaging materials.

(5) Maintaining gloves, if they are used in food handling, in an

intact, clean, and sanitary condition. The gloves should be of an

impermeable material.

(6) Wearing, where appropriate, in an effective manner, hair nets,

headbands, caps, beard covers, or other effective hair restraints.

(7) Storing clothing or other personal belongings in areas other

than where food is exposed or where equipment or utensils are washed.

(8) Confining the following to areas other than where food may be

exposed or where equipment or utensils are washed: eating food, chewing

gum, drinking beverages, or using tobacco.

(9) Taking any other necessary precautions to protect against

contamination of food, food-contact surfaces, or food-packaging

materials with microorganisms or foreign substances including, but not

limited to, perspiration, hair, cosmetics, tobacco, chemicals, and

medicines applied to the skin.

(c) Education and training. Personnel responsible for identifying

sanitation failures or food contamination should have a background of

education or experience, or a combination thereof, to provide a level of

competency necessary for production of clean and safe food. Food

handlers and supervisors should receive appropriate training in proper

food handling techniques and food-protection principles and should be

informed of the danger of poor personal hygiene and insanitary

practices.

(d) Supervision. Responsibility for assuring compliance by all

personnel with all requirements of this part shall be clearly assigned

to competent supervisory personnel.

[51 FR 24475, June 19, 1986, as amended at 54 FR 24892, June 12, 1989]

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.19]

[Page 217]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart A--General Provisions

Sec. 110.19  Exclusions.

(a) The following operations are not subject to this part:

Establishments engaged solely in the harvesting, storage, or

distribution of one or more ``raw agricultural commodities,'' as defined

in section 201(r) of the act, which are ordinarily cleaned, prepared,

treated, or otherwise processed before being marketed to the consuming

public.

(b) FDA, however, will issue special regulations if it is necessary

to cover these excluded operations.

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.20]

[Page 217-218]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart B--Buildings and Facilities

Sec. 110.20  Plant and grounds.

(a) Grounds. The grounds about a food plant under the control of the

operator shall be kept in a condition that will protect against the

contamination of food. The methods for adequate maintenance of grounds

include, but are not limited to:

(1) Properly storing equipment, removing litter and waste, and

cutting weeds or grass within the immediate vicinity of the plant

buildings or structures that may constitute an attractant, breeding

place, or harborage for pests.

(2) Maintaining roads, yards, and parking lots so that they do not

constitute a source of contamination in areas where food is exposed.

(3) Adequately draining areas that may contribute contamination to

food by seepage, foot-borne filth, or providing a breeding place for

pests.

(4) Operating systems for waste treatment and disposal in an

adequate manner so that they do not constitute a source of contamination

in areas where food is exposed.

If the plant grounds are bordered by grounds not under the operator's

control and not maintained in the manner described in paragraph (a) (1)

through (3) of this section, care shall be exercised in the plant by

inspection, extermination, or other means to exclude pests, dirt, and

filth that may be a source of food contamination.

[[Page 218]]

(b) Plant construction and design. Plant buildings and structures

shall be suitable in size, construction, and design to facilitate

maintenance and sanitary operations for food-manufacturing purposes. The

plant and facilities shall:

(1) Provide sufficient space for such placement of equipment and

storage of materials as is necessary for the maintenance of sanitary

operations and the production of safe food.

(2) Permit the taking of proper precautions to reduce the potential

for contamination of food, food-contact surfaces, or food-packaging

materials with microorganisms, chemicals, filth, or other extraneous

material. The potential for contamination may be reduced by adequate

food safety controls and operating practices or effective design,

including the separation of operations in which contamination is likely

to occur, by one or more of the following means: location, time,

partition, air flow, enclosed systems, or other effective means.

(3) Permit the taking of proper precautions to protect food in

outdoor bulk fermentation vessels by any effective means, including:

(i) Using protective coverings.

(ii) Controlling areas over and around the vessels to eliminate

harborages for pests.

(iii) Checking on a regular basis for pests and pest infestation.

(iv) Skimming the fermentation vessels, as necessary.

(4) Be constructed in such a manner that floors, walls, and ceilings

may be adequately cleaned and kept clean and kept in good repair; that

drip or condensate from fixtures, ducts and pipes does not contaminate

food, food-contact surfaces, or food-packaging materials; and that

aisles or working spaces are provided between equipment and walls and

are adequately unobstructed and of adequate width to permit employees to

perform their duties and to protect against contaminating food or food-

contact surfaces with clothing or personal contact.

(5) Provide adequate lighting in hand-washing areas, dressing and

locker rooms, and toilet rooms and in all areas where food is examined,

processed, or stored and where equipment or utensils are cleaned; and

provide safety-type light bulbs, fixtures, skylights, or other glass

suspended over exposed food in any step of preparation or otherwise

protect against food contamination in case of glass breakage.

(6) Provide adequate ventilation or control equipment to minimize

odors and vapors (including steam and noxious fumes) in areas where they

may contaminate food; and locate and operate fans and other air-blowing

equipment in a manner that minimizes the potential for contaminating

food, food-packaging materials, and food-contact surfaces.

(7) Provide, where necessary, adequate screening or other protection

against pests.

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.35]

[Page 218-219]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart B--Buildings and Facilities

Sec. 110.35  Sanitary operations.

(a) General maintenance. Buildings, fixtures, and other physical

facilities of the plant shall be maintained in a sanitary condition and

shall be kept in repair sufficient to prevent food from becoming

adulterated within the meaning of the act. Cleaning and sanitizing of

utensils and equipment shall be conducted in a manner that protects

against contamination of food, food-contact surfaces, or food-packaging

materials.

(b) Substances used in cleaning and sanitizing; storage of toxic

materials. (1) Cleaning compounds and sanitizing agents used in cleaning

and sanitizing procedures shall be free from undesirable microorganisms

and shall be safe and adequate under the conditions of use. Compliance

with this requirement may be verified by any effective means including

purchase of these substances under a supplier's guarantee or

certification, or examination of these substances for contamination.

Only the following toxic materials may be used or stored in a plant

where food is processed or exposed:

(i) Those required to maintain clean and sanitary conditions;

(ii) Those necessary for use in laboratory testing procedures;

(iii) Those necessary for plant and equipment maintenance and

operation; and

(iv) Those necessary for use in the plant's operations.

[[Page 219]]

(2) Toxic cleaning compounds, sanitizing agents, and pesticide

chemicals shall be identified, held, and stored in a manner that

protects against contamination of food, food-contact surfaces, or food-

packaging materials. All relevant regulations promulgated by other

Federal, State, and local government agencies for the application, use,

or holding of these products should be followed.

(c) Pest control. No pests shall be allowed in any area of a food

plant. Guard or guide dogs may be allowed in some areas of a plant if

the presence of the dogs is unlikely to result in contamination of food,

food-contact surfaces, or food-packaging materials. Effective measures

shall be taken to exclude pests from the processing areas and to protect

against the contamination of food on the premises by pests. The use of

insecticides or rodenticides is permitted only under precautions and

restrictions that will protect against the contamination of food, food-

contact surfaces, and food-packaging materials.

(d) Sanitation of food-contact surfaces. All food-contact surfaces,

including utensils and food-contact surfaces of equipment, shall be

cleaned as frequently as necessary to protect against contamination of

food.

(1) Food-contact surfaces used for manufacturing or holding low-

moisture food shall be in a dry, sanitary condition at the time of use.

When the surfaces are wet-cleaned, they shall, when necessary, be

sanitized and thoroughly dried before subsequent use.

(2) In wet processing, when cleaning is necessary to protect against

the introduction of microorganisms into food, all food-contact surfaces

shall be cleaned and sanitized before use and after any interruption

during which the food-contact surfaces may have become contaminated.

Where equipment and utensils are used in a continuous production

operation, the utensils and food-contact surfaces of the equipment shall

be cleaned and sanitized as necessary.

(3) Non-food-contact surfaces of equipment used in the operation of

food plants should be cleaned as frequently as necessary to protect

against contamination of food.

(4) Single-service articles (such as utensils intended for one-time

use, paper cups, and paper towels) should be stored in appropriate

containers and shall be handled, dispensed, used, and disposed of in a

manner that protects against contamination of food or food-contact

surfaces.

(5) Sanitizing agents shall be adequate and safe under conditions of

use. Any facility, procedure, or machine is acceptable for cleaning and

sanitizing equipment and utensils if it is established that the

facility, procedure, or machine will routinely render equipment and

utensils clean and provide adequate cleaning and sanitizing treatment.

(e) Storage and handling of cleaned portable equipment and utensils.

Cleaned and sanitized portable equipment with food-contact surfaces and

utensils should be stored in a location and manner that protects food-

contact surfaces from contamination.

[51 FR 24475, June 19, 1986, as amended at 54 FR 24892, June 12, 1989]

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.37]

[Page 219-220]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart B--Buildings and Facilities

Sec. 110.37  Sanitary facilities and controls.

Each plant shall be equipped with adequate sanitary facilities and

accommodations including, but not limited to:

(a) Water supply. The water supply shall be sufficient for the

operations intended and shall be derived from an adequate source. Any

water that contacts food or food-contact surfaces shall be safe and of

adequate sanitary quality. Running water at a suitable temperature, and

under pressure as needed, shall be provided in all areas where required

for the processing of food, for the cleaning of equipment, utensils, and

food-packaging materials, or for employee sanitary facilities.

(b) Plumbing. Plumbing shall be of adequate size and design and

adequately installed and maintained to:

(1) Carry sufficient quantities of water to required locations

throughout the plant.

(2) Properly convey sewage and liquid disposable waste from the

plant.

(3) Avoid constituting a source of contamination to food, water

supplies, equipment, or utensils or creating an unsanitary condition.

[[Page 220]]

(4) Provide adequate floor drainage in all areas where floors are

subject to flooding-type cleaning or where normal operations release or

discharge water or other liquid waste on the floor.

(5) Provide that there is not backflow from, or cross-connection

between, piping systems that discharge waste water or sewage and piping

systems that carry water for food or food manufacturing.

(c) Sewage disposal. Sewage disposal shall be made into an adequate

sewerage system or disposed of through other adequate means.

(d) Toilet facilities. Each plant shall provide its employees with

adequate, readily accessible toilet facilities. Compliance with this

requirement may be accomplished by:

(1) Maintaining the facilities in a sanitary condition.

(2) Keeping the facilities in good repair at all times.

(3) Providing self-closing doors.

(4) Providing doors that do not open into areas where food is

exposed to airborne contamination, except where alternate means have

been taken to protect against such contamination (such as double doors

or positive air-flow systems).

(e) Hand-washing facilities. Hand-washing facilities shall be

adequate and convenient and be furnished with running water at a

suitable temperature. Compliance with this requirement may be

accomplished by providing:

(1) Hand-washing and, where appropriate, hand-sanitizing facilities

at each location in the plant where good sanitary practices require

employees to wash and/or sanitize their hands.

(2) Effective hand-cleaning and sanitizing preparations.

(3) Sanitary towel service or suitable drying devices.

(4) Devices or fixtures, such as water control valves, so designed

and constructed to protect against recontamination of clean, sanitized

hands.

(5) Readily understandable signs directing employees handling

unproteced food, unprotected food-packaging materials, of food-contact

surfaces to wash and, where appropriate, sanitize their hands before

they start work, after each absence from post of duty, and when their

hands may have become soiled or contaminated. These signs may be posted

in the processing room(s) and in all other areas where employees may

handle such food, materials, or surfaces.

(6) Refuse receptacles that are constructed and maintained in a

manner that protects against contamination of food.

(f) Rubbish and offal disposal. Rubbish and any offal shall be so

conveyed, stored, and disposed of as to minimize the development of

odor, minimize the potential for the waste becoming an attractant and

harborage or breeding place for pests, and protect against contamination

of food, food-contact surfaces, water supplies, and ground surfaces.

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.40]

[Page 220-221]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart C--Equipment

Sec. 110.40  Equipment and utensils.

(a) All plant equipment and utensils shall be so designed and of

such material and workmanship as to be adequately cleanable, and shall

be properly maintained. The design, construction, and use of equipment

and utensils shall preclude the adulteration of food with lubricants,

fuel, metal fragments, contaminated water, or any other contaminants.

All equipment should be so installed and maintained as to facilitate the

cleaning of the equipment and of all adjacent spaces. Food-contact

surfaces shall be corrosion-resistant when in contact with food. They

shall be made of nontoxic materials and designed to withstand the

environment of their intended use and the action of food, and, if

applicable, cleaning compounds and sanitizing agents. Food-contact

surfaces shall be maintained to protect food from being contaminated by

any source, including unlawful indirect food additives.

(b) Seams on food-contact surfaces shall be smoothly bonded or

maintained so as to minimize accumulation of food particles, dirt, and

organic matter and thus minimize the opportunity for growth of

microorganisms.

(c) Equipment that is in the manufacturing or food-handling area and

that does not come into contact with food shall be so constructed that

it can be kept in a clean condition.

[[Page 221]]

(d) Holding, conveying, and manufacturing systems, including

gravimetric, pneumatic, closed, and automated systems, shall be of a

design and construction that enables them to be maintained in an

appropriate sanitary condition.

(e) Each freezer and cold storage compartment used to store and hold

food capable of supporting growth of microorganisms shall be fitted with

an indicating thermometer, temperature-measuring device, or temperature-

recording device so installed as to show the temperature accurately

within the compartment, and should be fitted with an automatic control

for regulating temperature or with an automatic alarm system to indicate

a significant temperature change in a manual operation.

(f) Instruments and controls used for measuring, regulating, or

recording temperatures, pH, acidity, water activity, or other conditions

that control or prevent the growth of undesirable microorganisms in food

shall be accurate and adequately maintained, and adequate in number for

their designated uses.

(g) Compressed air or other gases mechanically introduced into food

or used to clean food-contact surfaces or equipment shall be treated in

such a way that food is not contaminated with unlawful indirect food

additives.

Subpart D [Reserved]

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.80]

[Page 221-224]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart E--Production and Process Controls

Sec. 110.80  Processes and controls.

All operations in the receiving, inspecting, transporting,

segregating, preparing, manufacturing, packaging, and storing of food

shall be conducted in accordance with adequate sanitation principles.

Appropriate quality control operations shall be employed to ensure that

food is suitable for human consumption and that food-packaging materials

are safe and suitable. Overall sanitation of the plant shall be under

the supervision of one or more competent individuals assigned

responsibility for this function. All reasonable precautions shall be

taken to ensure that production procedures do not contribute

contamination from any source. Chemical, microbial, or extraneous-

material testing procedures shall be used where necessary to identify

sanitation failures or possible food contamination. All food that has

become contaminated to the extent that it is adulterated within the

meaning of the act shall be rejected, or if permissible, treated or

processed to eliminate the contamination.

(a) Raw materials and other ingredients. (1) Raw materials and other

ingredients shall be inspected and segregated or otherwise handled as

necessary to ascertain that they are clean and suitable for processing

into food and shall be stored under conditions that will protect against

contamination and minimize deterioration. Raw materials shall be washed

or cleaned as necessary to remove soil or other contamination. Water

used for washing, rinsing, or conveying food shall be safe and of

adequate sanitary quality. Water may be reused for washing, rinsing, or

conveying food if it does not increase the level of contamination of the

food. Containers and carriers of raw materials should be inspected on

receipt to ensure that their condition has not contributed to the

contamination or deterioration of food.

(2) Raw materials and other ingredients shall either not contain

levels of microorganisms that may produce food poisoning or other

disease in humans, or they shall be pasteurized or otherwise treated

during manufacturing operations so that they no longer contain levels

that would cause the product to be adulterated within the meaning of the

act. Compliance with this requirement may be verified by any effective

means, including purchasing raw materials and other ingredients under a

supplier's guarantee or certification.

(3) Raw materials and other ingredients susceptible to contamination

with aflatoxin or other natural toxins shall comply with current Food

and Drug Administration regulations and action levels for poisonous or

deleterious substances before these materials or ingredients are

incorporated into finished food. Compliance with this requirement may be

accomplished by purchasing raw materials and other ingredients under a

supplier's guarantee or

[[Page 222]]

certification, or may be verified by analyzing these materials and

ingredients for aflatoxins and other natural toxins.

(4) Raw materials, other ingredients, and rework susceptible to

contamination with pests, undesirable microorganisms, or extraneous

material shall comply with applicable Food and Drug Administration

regulations and defect action levels for natural or unavoidable defects

if a manufacturer wishes to use the materials in manufacturing food.

Compliance with this requirement may be verified by any effective means,

including purchasing the materials under a supplier's guarantee or

certification, or examination of these materials for contamination.

(5) Raw materials, other ingredients, and rework shall be held in

bulk, or in containers designed and constructed so as to protect against

contamination and shall be held at such temperature and relative

humidity and in such a manner as to prevent the food from becoming

adulterated within the meaning of the act. Material scheduled for rework

shall be identified as such.

(6) Frozen raw materials and other ingredients shall be kept frozen.

If thawing is required prior to use, it shall be done in a manner that

prevents the raw materials and other ingredients from becoming

adulterated within the meaning of the act.

(7) Liquid or dry raw materials and other ingredients received and

stored in bulk form shall be held in a manner that protects against

contamination.

(b) Manufacturing operations. (1) Equipment and utensils and

finished food containers shall be maintained in an acceptable condition

through appropriate cleaning and sanitizing, as necessary. Insofar as

necessary, equipment shall be taken apart for thorough cleaning.

(2) All food manufacturing, including packaging and storage, shall

be conducted under such conditions and controls as are necessary to

minimize the potential for the growth of microorganisms, or for the

contamination of food. One way to comply with this requirement is

careful monitoring of physical factors such as time, temperature,

humidity, aw, pH, pressure, flow rate, and manufacturing

operations such as freezing, dehydration, heat processing,

acidification, and refrigeration to ensure that mechanical breakdowns,

time delays, temperature fluctuations, and other factors do not

contribute to the decomposition or contamination of food.

(3) Food that can support the rapid growth of undesirable

microorganisms, particularly those of public health significance, shall

be held in a manner that prevents the food from becoming adulterated

within the meaning of the act. Compliance with this requirement may be

accomplished by any effective means, including:

(i) Maintaining refrigerated foods at 45  deg.F (7.2  deg.C) or

below as appropriate for the particular food involved.

(ii) Maintaining frozen foods in a frozen state.

(iii) Maintaining hot foods at 140  deg.F (60  deg.C) or above.

(iv) Heat treating acid or acidified foods to destroy mesophilic

microorganisms when those foods are to be held in hermetically sealed

containers at ambient temperatures.

(4) Measures such as sterilizing, irradiating, pasteurizing,

freezing, refrigerating, controlling pH or controlling aw

that are taken to destroy or prevent the growth of undesirable

microorganisms, particularly those of public health significance, shall

be adequate under the conditions of manufacture, handling, and

distribution to prevent food from being adulterated within the meaning

of the act.

(5) Work-in-process shall be handled in a manner that protects

against contamination.

(6) Effective measures shall be taken to protect finished food from

contamination by raw materials, other ingredients, or refuse. When raw

materials, other ingredients, or refuse are unprotected, they shall not

be handled simultaneously in a receiving, loading, or shipping area if

that handling could result in contaminated food. Food transported by

conveyor shall be protected against contamination as necessary.

(7) Equipment, containers, and utensils used to convey, hold, or

store raw materials, work-in-process, rework, or food shall be

constructed, handled, and maintained during manufacturing or

[[Page 223]]

storage in a manner that protects against contamination.

(8) Effective measures shall be taken to protect against the

inclusion of metal or other extraneous material in food. Compliance with

this requirement may be accomplished by using sieves, traps, magnets,

electronic metal detectors, or other suitable effective means.

(9) Food, raw materials, and other ingredients that are adulterated

within the meaning of the act shall be disposed of in a manner that

protects against the contamination of other food. If the adulterated

food is capable of being reconditioned, it shall be reconditioned using

a method that has been proven to be effective or it shall be reexamined

and found not to be adulterated within the meaning of the act before

being incorporated into other food.

(10) Mechanical manufacturing steps such as washing, peeling,

trimming, cutting, sorting and inspecting, mashing, dewatering, cooling,

shredding, extruding, drying, whipping, defatting, and forming shall be

performed so as to protect food against contamination. Compliance with

this requirement may be accomplished by providing adequate physical

protection of food from contaminants that may drip, drain, or be drawn

into the food. Protection may be provided by adequate cleaning and

sanitizing of all food-contact surfaces, and by using time and

temperature controls at and between each manufacturing step.

(11) Heat blanching, when required in the preparation of food,

should be effected by heating the food to the required temperature,

holding it at this temperature for the required time, and then either

rapidly cooling the food or passing it to subsequent manufacturing

without delay. Thermophilic growth and contamination in blanchers should

be minimized by the use of adequate operating temperatures and by

periodic cleaning. Where the blanched food is washed prior to filling,

water used shall be safe and of adequate sanitary quality.

(12) Batters, breading, sauces, gravies, dressings, and other

similar preparations shall be treated or maintained in such a manner

that they are protected against contamination. Compliance with this

requirement may be accomplished by any effective means, including one or

more of the following:

(i) Using ingredients free of contamination.

(ii) Employing adequate heat processes where applicable.

(iii) Using adequate time and temperature controls.

(iv) Providing adequate physical protection of components from

contaminants that may drip, drain, or be drawn into them.

(v) Cooling to an adequate temperature during manufacturing.

(vi) Disposing of batters at appropriate intervals to protect

against the growth of microorganisms.

(13) Filling, assembling, packaging, and other operations shall be

performed in such a way that the food is protected against

contamination. Compliance with this requirement may be accomplished by

any effective means, including:

(i) Use of a quality control operation in which the critical control

points are identified and controlled during manufacturing.

(ii) Adequate cleaning and sanitizing of all food-contact surfaces

and food containers.

(iii) Using materials for food containers and food- packaging

materials that are safe and suitable, as defined in Sec. 130.3(d) of

this chapter.

(iv) Providing physical protection from contamination, particularly

airborne contamination.

(v) Using sanitary handling procedures.

(14) Food such as, but not limited to, dry mixes, nuts, intermediate

moisture food, and dehydrated food, that relies on the control of

aw for preventing the growth of undesirable microorganisms

shall be processed to and maintained at a safe moisture level.

Compliance with this requirement may be accomplished by any effective

means, including employment of one or more of the following practices:

(i) Monitoring the aw of food.

(ii) Controlling the soluble solids-water ratio in finished food.

(iii) Protecting finished food from moisture pickup, by use of a

moisture barrier or by other means, so that the

[[Page 224]]

aw of the food does not increase to an unsafe level.

(15) Food such as, but not limited to, acid and acidified food, that

relies principally on the control of pH for preventing the growth of

undesirable microorganisms shall be monitored and maintained at a pH of

4.6 or below. Compliance with this requirement may be accomplished by

any effective means, including employment of one or more of the

following practices:

(i) Monitoring the pH of raw materials, food in process, and

finished food.

(ii) Controlling the amount of acid or acidified food added to low-

acid food.

(16) When ice is used in contact with food, it shall be made from

water that is safe and of adequate sanitary quality, and shall be used

only if it has been manufactured in accordance with current good

manufacturing practice as outlined in this part.

(17) Food-manufacturing areas and equipment used for manufacturing

human food should not be used to manufacture nonhuman food-grade animal

feed or inedible products, unless there is no reasonable possibility for

the contamination of the human food.

[51 FR 24475, June 19, 1986, as amended at 65 FR 56479, Sept. 19, 2000]

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.93]

[Page 224]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart E--Production and Process Controls

Sec. 110.93  Warehousing and distribution.

Storage and transportation of finished food shall be under

conditions that will protect food against physical, chemical, and

microbial contamination as well as against deterioration of the food and

the container.

Subpart F [Reserved]

[Code of Federal Regulations]

[Title 21, Volume 2]

[Revised as of April 1, 2003]

From the U.S. Government Printing Office via GPO Access

[CITE: 21CFR110.110]

[Page 224]

TITLE 21--FOOD AND DRUGS

CHAPTER I--FOOD AND DRUG ADMINISTRATION, DEPARTMENT OF HEALTH AND HUMAN

SERVICES (CONTINUED)

PART 110--CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PACKING,

OR HOLDING HUMAN FOOD--Table of Contents

Subpart G--Defect Action Levels

Sec. 110.110  Natural or unavoidable defects in food for human use that present no health hazard.

(a) Some foods, even when produced under current good manufacturing

practice, contain natural or unavoidable defects that at low levels are

not hazardous to health. The Food and Drug Administration establishes

maximum levels for these defects in foods produced under current good

manufacturing practice and uses these levels in deciding whether to

recommend regulatory action.

(b) Defect action levels are established for foods whenever it is

necessary and feasible to do so. These levels are subject to change upon

the development of new technology or the availability of new

information.

(c) Compliance with defect action levels does not excuse violation

of the requirement in section 402(a)(4) of the act that food not be

prepared, packed, or held under unsanitary conditions or the

requirements in this part that food manufacturers, distributors, and

holders shall observe current good manufacturing practice. Evidence

indicating that such a violation exists causes the food to be

adulterated within the meaning of the act, even though the amounts of

natural or unavoidable defects are lower than the currently established

defect action levels. The manufacturer, distributor, and holder of food

shall at all times utilize quality control operations that reduce

natural or unavoidable defects to the lowest level currently feasible.

(d) The mixing of a food containing defects above the current defect

action level with another lot of food is not permitted and renders the

final food adulterated within the meaning of the act, regardless of the

defect level of the final food.

(e) A compilation of the current defect action levels for natural or

unavoidable defects in food for human use that present no health hazard

may be obtained upon request from the Center for Food Safety and Applied

Nutrition (HFS-565), Food and Drug Administration, 5100 Paint Branch

Pkwy., College Park, MD 20740.

[51 FR 24475, June 19, 1986, as amended at 61 FR 14480, Apr. 2, 1996; 66

FR 56035, Nov. 6, 2001]